ACOUSTICAL SURFACES, INC.

NOISE S.T.O.P. – INTERIOR ACOUSTICAL - SOUNDPROOF WINDOWS

SECTION 08567 – SOUND CONTROL WINDOWS
PART 1 -  GENERAL

1.1 RELATED DOCUMENTS

A. Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

1.2 SUMMARY

A. This Section includes sound control windows with STC ratings of [45] [49] [56].
1.3 SUBMITTALS

A. Product Data: Submit manufacturer's product data and installation instructions.

B. Test Reports: Upon request submit certified test reports from recognized test laboratories.
C. Certificates: Submit manufacturer's certificate that products meet or exceed specified requirements.

1.4 DELIVERY, STORAGE & HANDLING

A. Delivery: Deliver material in the manufacturer’s original, unopened, undamaged containers with identification labels intact.

B. Provide labels indicating brand name, source of procurement, style, size and thickness.
C. Storage and Protection: Store materials protected from exposure to harmful environmental conditions and at temperature and humidity conditions recommended by the manufacturer.

1.5 WARRANTY

A. Warranty Period:  Two years.
PART 2 -  PRODUCTS

2.1 MANUFACTURERS

A. Sound Control Windows:

1. Noise S.T.O.P. Interior Acoustical – Soundproof Windows by Acoustical Surfaces Inc. (Basis of Design)
a. 123 Columbia Court North Suite 210.

b. Chaska, MN  55318.
c. (800) 448 - 0121.
d. www.AcousticalSurfaces.com
2. Or equal.

2.2 SOUND CONTROL WINDOWS
A. Product: Model Studio 4.

1. STC rating: 45.

2. 4 degree angled glass for max acoustical performance. [Optional flat glass configuration.]
3. Size (clear opening): [Max size of 4' by 8' rectangle] [Max size of 5' by 5' square] [Larger units available with mullions] [Custom sizes] [As indicated on Drawing].
4. Tint: Optional [tinted] [one-way mirror pane] [safety glass] [Custom glass].

5. Glazing: 1" insulated glass unit consists of 2 pieces of 1/4" laminated glass.

6. Frame: 4-1/2" anodized aluminum frame.

B. Product: Model Studio 5.

1. STC rating: 49.

2. Flat glass configuration.

3. Size (clear opening): [Max size of 5' by 10' rectangle] [Larger units available with mullions] [Custom sizes] [As indicated on Drawing].
4. Tint: Optional [Tinted] [One-way mirror pane] [Safety glass] [Custom glass].

5. Glazing: 3/8" laminated glass with air cavity and 1/4" laminated glass.

6. Frame: 4-1/2" anodized aluminum frame.

C. Product: Model Studio 6.

1. STC rating: 56.

2. 4 degree angled glass for max acoustical performance. [Optional flat glass configuration].
3. Size (clear opening): [Max size of 4' by 8' rectangle] [Max size of 5' by 5' square] [Larger units available with mullions] [Custom sizes] [As indicated on Drawing].
4. Tint: Optional [Tinted] [One-way mirror pane] [Safety glass] [Custom glass].

5. Glazing: 1” insulated glass unit consists of 2 pieces of 1/4" laminated glass plus an air cavity and an additional pane of 3/8 " laminated glass.

6. Frame: 6-3/8" anodized aluminum frame.

D. Product: Model Studio 8.
1. STC rating: 59.

2. 4 degree angled glass for max acoustical performance. [Optional flat glass configuration].
3. Size (clear opening): [Max size of 4' by 8' rectangle] [Max size of 5' by 5' square] [Larger units available with mullions] [Custom sizes] [As indicated on Drawing].
4. Tint: Optional [Tinted] [One-way mirror pane] [Safety glass] [Custom glass].

5. Glazing: 1” insulated glass unit consists of 2 pieces of 1/4" laminated glass plus an air cavity and an additional pane of 3/8 " laminated glass.

6. Frame: 12-7/8" anodized aluminum frame.

E. Window Frame:

1. Anodized aluminum.

a. Finish: [Clear] [Bronze] [As indicated on Drawings].
2. Angled glass for maximum acoustical performance.

3. Acoustically treated frames and seals.

4. Class A-1 ASTM E-84 Nonflammable Sound Silencer™ acoustical frame insert.

5. Fits any wall thickness by insetting or offsetting acoustical window.

F. Applications: Recording studios, commercial windows, industrial, military, testing labs, victim/interrogation rooms, and soundproof rooms.
PART 3 -  EXECUTION

3.1 EXAMINATION

A. Examine openings, substrates, anchorage, and conditions.

B. Immediately inspect shipment for damage during transit i.e., damage caused by fork lifts, stacking, water stains etc. and disclose to delivery driver prior to signing for receipt.

3.2 INSTALLATION

A. Manufacturer’s Instructions:

1. Comply with the instructions and recommendations of the window manufacturer.

2. Installation manual is available from manufacturer’s website.

B. Shipping:

1. Glass shipped separately to avoid breakage.

2. Easy snap in glass installation on job site.

3.3 CLEANING

A. Clean aluminum surfaces immediately after installing windows.  Avoid damaging protective coatings and finishes.  Remove excess sealants, glazing materials, dirt, and other substances.

END OF SECTION 08567
SOUND CONTROL WINDOWS
08567 - 1

